Walkera Genius CP Devo 7 - Step by Step Setup Guide

This setup will give you 8-9* +/- pitch.
If you want more or less pitch turn up or down the pitch travel.
DO NOT set Rudder travel adjustment more than 100%. The tail will blow out
hard with any setting over 100%.

Copy a model in the Devo 7:
At the default screen Press ENT, select MODEL on the top - Press ENT, select COPY - Press ENT, for SOURCE select select GENIS - Press ENT, for DEST select MOD 2 - Press ENT.


Intermediate to Advanced Settings
See picture below for the RX factory pot settings.
Verify the settings after you are done before you fly the Genius CP.

SWASH - 1-NRM
STMOD - MODE2 if you want the throttle on the left stick
REVERS - None, all normal
TRVAD - Pitch L & H = 65% to 70%
SUBTR - ALL 0.0%
DREXP - Set D/R and EXPO to your personal needs/preference. Here is what I use.

ELEV D/R 80% EXP +30% > Switch position 1
ELEV D/R 100% EXP +30% > Switch position 0
AILE D/R - 80%, EXP +30% > Switch position 1
AILE D/R - 100%, EXP +30% > Switch position 0
RUDD D/R - 100%, EXP -2% > Switch position 0 & 1

THHLD - STATE - ACT 
POSIT - 0.0%
Set throttle trim to center!
It looks like Genius has firmware to run like this.

GYRO - MODE- AUTO 

Set as high as you can go without getting tail wag. 

NORM - 82.0%
ST 1 - 82.0%
ST 2 - 82.0%
THHLD - This doesn't function on the Genius. Leave it where it is set to.

THCRV

To be able to set a value for a Throttle Curve you need to change STATE - INH to ACT

MODE - NORM

Set EXP to OFF

SRVHD - It appears this stops the servos or throttle on the helicopter from moving while you are in the program menu. When you exit they function again. I selected YES then pressed ENT. If you want to work with the values and have throttle and servos working select NO then press ENT for SRVHD.

PL = 0.0%
P1 = INH
P2 - INH
PM = 100.0%
P3 = INH
P4 = INH
PH = 100.0%

MODE - ST-1

Set EXP to OFF
PL = 100.0%
P1 = INH
P2 = INH
PM = 100.0%
P3 = INH
P4 = INH
PH = 100.0%

MODE - ST-2

Set EXP to OFF
PL = 100.0%
P1 = INH
P2 = INH
PM = 100.0%
P3 = INH
P4 = INH
PH = 100.0%

PTCRV

To be able to set a value for a Picth Curve you need to change STATE - INH to ACT

MODE - NORM

Set EXP to OFF

SRVHD - It appears this stops the servos or throttle on the helicopter from moving while you are in the program menu. When you exit they function again.
I selected YES then pressed ENT. If you want to work with the values and have throttle and servos working select NO then press ENT for SRVHD.

PL = -35%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%

MODE - ST-1

Set EXP to OFF
PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%

MODE - ST-2

Set EXP to OFF
PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%


MODE - THHLD

Set EXP to OFF
PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%

----------------------------------------------------------------------------------------------------------

Beginner Settings

See picture below for the RX factory pot settings.

Note: With these beginner settings there will be a height change when you switch back and fourth between Normal and Stunt modes.
The throttle curves have been tamed down for beginners. The pitch travel has also been tamed down for beginners. Normal mode will give you about 2-3* negative pitch.
More EXPO has been added and more D/R is used.

Verify the settings after you are done before you fly the Genius CP.

SWASH - 1-NRM
STMOD - MODE2 if you want the throttle on the left stick
REVERS - None, all normal
TRVAD - Pitch L & H = 60% 

SUBTR - ALL 0.0%
DREXP - Set D/R and EXPO to your personal needs/preference. Here is what I use.

ELEV D/R 70% EXP +35% > Switch position 1
ELEV D/R 100% EXP +35% > Switch position 0
AILE D/R - 70%, EXP +35% > Switch position 1
AILE D/R - 100%, EXP +35% > Switch position 0
RUDD D/R - 100%, EXP +25% > Switch position 0 & 1

THHLD - STATE - ACT 
POSIT - 0.0%
Set throttle trim to center!
It looks like Genius has firmware to run like this.

GYRO - MODE- AUTO 
Set as high as you can go without getting tail wag. If you get tail wag with these numbers lower them
all equally EX: 78.0%, 78.0%, 78.0%

NORM - 82.0%
ST 1 - 82.0%
ST 2 - 82.0%
THHLD - This doesn't function on the Genius. Leave it where it is set to.

THCRV

To be able to set a value for a Throttle Curve you need to change STATE - INH to ACT

MODE - NORM

Set EXP to OFF

SRVHD - It appears this stops the servos or throttle on the helicopter from moving while you are in the program menu. When you exit they function again.
I selected YES then pressed ENT. If you want to work with the values and have throttle and servos working select NO then press ENT for SRVHD.

PL = 0.0%
P1 = INH
P2 - INH
PM = INH
P3 = INH
P4 = INH
PH = 100.0%

MODE - ST-1

Set EXP to OFF
PL = 100.0%
P1 = INH
P2 = INH
PM = 80.0%
P3 = INH
P4 = INH
PH = 100.0%

MODE - ST-2

Set EXP to OFF
PL = 100.0%
P1 = INH
P2 = INH
PM = 100.0%
P3 = INH
P4 = INH
PH = 100.0%

PTCRV

To be able to set a value for a Picth Curve you need to change STATE - INH to ACT

MODE - NORM

Set EXP to OFF

SRVHD - It appears this stops the servos or throttle on the helicopter from moving while you are in the program menu. When you exit they function again.
I selected YES then pressed ENT. If you want to work with the values and have throttle and servos working select NO then press ENT for SRVHD.

PL = -35%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%

MODE - ST-1

Set EXP to OFF
PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%

MODE - ST-2

Set EXP to OFF

PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%


MODE - THHLD

Set EXP to OFF

PL = -100%
P1 = INH
P2 = INH
PM = 0%
P3 = INH
P4 = INH
PH = +100%